This is an example project to demonstrate how you can receive a new application image while running from the main application. When the app has received the image, it will perform a system reset to let the bootloader activate the new app (i.e., verify update then copy the image from bank 1 to bank 0).

The project is based on the heart rate sensor example in SDK 15.3.0 (ble_app_hrs) and includes the serial UART DFU transport from the UART bootloader.

Prerequisites for testing DFU

- Nrfutil

- Segger Embedded Studio

- nRF52832 development kit

Testing

- Open the ble_app_hrs_w_serial_dfu project in Segger embedded studio (SES) and program it to the board.

- Run the “program_bl_sd_bl_settings” script to program the bootloader, Softdevice, and the bootloader settings which validates the application.

- Start a debug session in SES to view the debug log

- Check which VCOM port number is assigned to the 52DK and update the port number used in the “serial_dfu” script (nrfutil dfu serial -pkg dfu_app_package.zip -p COM<port number>).

- Run the serial_dfu script to start serial DFU

Notes

- Zigbee/Thread SDK use the same bootloader for background DFU and has undergone more thorough testing. Those may be used as a reference in addition to this project

- Only application update is supported. Bootloader and Softdevice must be updated through the bootloader

- For custom DFU transports: replace nrf_dfu_serial.c and nrf_dfu_serial_uart.c in the application project.

- Nrf_dfu_flash.c was modified because it used the same variable name for the fstorage instance as FDS uses.

Expected log output before and after successful DFU

Memory layout